3-DOF Robot Arm
Product of Exponentials (POE) & Forward Kinematics

Documentation of the work completed so far — from space screws to body screws

# 1. Robot Schematic

The following schematic is the robot model used in this derivation. The physical chain is θ₁ → l₁ → θ₂ → l₂ → θ₃ → l₃ → end effector.

Figure 1. 3-DOF robot arm schematic.

# 2. Robot Dimensions and Convention

The link lengths used for the mathematical model are:

| Parameter | Meaning | Value |
| --- | --- | --- |
| l₁ | Base to Joint 2 | 0.30 m |
| l₂ | Joint 2 to Joint 3 | 0.40 m |
| l₃ | Joint 3 to end effector | 0.30 m |

The space frame is fixed at the robot base. The body frame is attached to the end effector. The home configuration is θ₁ = θ₂ = θ₃ = 0.

# 3. POE Building Blocks

## 3.1 Homogeneous Transformation Matrix

A homogeneous transformation contains orientation and position:

T = [ R  p ; 0  1 ]

R is the 3×3 rotation matrix and p is the 3×1 position vector.

## 3.2 Space Frame

The space frame is the fixed coordinate frame attached to the robot base. It does not move when the robot joints move.

## 3.3 Body Frame

The body frame is attached to the end effector and therefore moves with the robot.

## 3.4 qᵢ, ωᵢ and vᵢ

qᵢ is a point on joint i's rotation axis. ωᵢ is the unit vector along its rotation axis. For a revolute joint:

vᵢ = −ωᵢ × qᵢ

The 6×1 screw axis is:

Sᵢ = [ ωᵢ ; vᵢ ]

# 4. Space-Frame Screw Axes

The values obtained for the current model are:

## Joint 1

q₁ = [0, 0, 0]ᵀ

ω₁ = [0, 0, 1]ᵀ

v₁ = [0, 0, 0]ᵀ

S₁ = [0, 0, 1, 0, 0, 0]ᵀ

## Joint 2

q₂ = [0, 0, 0.30]ᵀ

ω₂ = [1, 0, 0]ᵀ

v₂ = [0, 0.30, 0]ᵀ

S₂ = [1, 0, 0, 0, 0.30, 0]ᵀ

## Joint 3

q₃ = [0.40, 0, 0.30]ᵀ

ω₃ = [1, 0, 0]ᵀ

v₃ = [0, 0.30, 0]ᵀ

S₃ = [1, 0, 0, 0, 0.30, 0]ᵀ

## 4.1 Total Space Screw Matrix

S = [ S₁  S₂  S₃ ] =
[ 0   1    1  ]
[ 0   0    0  ]
[ 1   0    0  ]
[ 0   0    0  ]
[ 0  0.30 0.30]
[ 0   0    0  ]

Important: with the current interpretation, S₂ and S₃ are identical. This is the result used in the work so far; the physical joint-3 axis should be checked against the schematic before using the model for a final numerical robot implementation.

# 5. Home Configuration M

The home configuration is:

M = T(0, 0, 0)

At zero angles, the chain gives z = l₁ = 0.30 m and x = l₂ + l₃ = 0.40 + 0.30 = 0.70 m.

Therefore:

pₑ = [0.70, 0, 0.30]ᵀ m

Assuming the end-effector axes are aligned with the space axes at home, R = I.

M =
[ 1  0  0  0.70 ]
[ 0  1  0  0    ]
[ 0  0  1  0.30 ]
[ 0  0  0  1    ]

## 5.1 Inverse of M

M⁻¹ =
[ 1  0  0  −0.70 ]
[ 0  1  0   0    ]
[ 0  0  1  −0.30 ]
[ 0  0  0   1    ]

# 6. Product of Exponentials Forward Kinematics

The space-frame POE formula calculates the end-effector pose from the joint angles:

T(θ₁, θ₂, θ₃) = exp([S₁]θ₁) exp([S₂]θ₂) exp([S₃]θ₃) M

For S = [ω; v], its 4×4 se(3) matrix is:

[S] = [ [ω]  v ; 0  0 ]

where [ω] is the skew-symmetric matrix:
[ω] = [ 0  −ωz  ωy ;  ωz  0  −ωx ; −ωy  ωx  0 ].

Once numerical values of θ₁, θ₂ and θ₃ are supplied, T gives the end-effector position and orientation expressed relative to the fixed space/base frame.

# 7. Body Screw Axes

Body screws describe the same physical joint motions, but expressed in the body/end-effector frame at the home configuration.

The conversion is:

Bᵢ = Ad(M⁻¹) Sᵢ

## 7.1 Adjoint Transformation

For T = [R p; 0 1]:
Ad(T) = [ R  0 ; [p]R  R ].

Because the current M has R = I, the conversion is especially simple. For M⁻¹, the linear component can be written as:

vᴮ = vˢ − p × ω

## 7.2 Body Screw Results

B₁ = [0, 0, 1, 0, 0.70, 0]ᵀ

B₂ = [1, 0, 0, 0, 0, 0]ᵀ

B₃ = [1, 0, 0, 0, 0, 0]ᵀ

## 7.3 Total Body Screw Matrix

B = [ B₁  B₂  B₃ ] =
[ 0   1   1 ]
[ 0   0   0 ]
[ 1   0   0 ]
[ 0   0   0 ]
[ 0.70 0  0 ]
[ 0   0   0 ]

# 8. Body-Frame POE

The equivalent body formulation is:

T(θ₁, θ₂, θ₃) = M exp([B₁]θ₁) exp([B₂]θ₂) exp([B₃]θ₃)

Notice the distinction: M⁻¹ is used to convert space screws into body screws, but the original M is used in the body POE forward-kinematics equation.

# 9. Space vs Body — Quick Comparison

|  | Space | Body |
| --- | --- | --- |
| Reference | Fixed base frame | End-effector frame |
| Screws | S₁, S₂, S₃ | B₁, B₂, B₃ |
| Conversion | — | Bᵢ = Ad(M⁻¹)Sᵢ |
| POE | exp(S₁θ₁) exp(S₂θ₂) exp(S₃θ₃) M | M exp(B₁θ₁) exp(B₂θ₂) exp(B₃θ₃) |

# 10. What Has Been Completed

Defined the space/base frame and body/end-effector frame.

Identified q₁, q₂, q₃ and ω₁, ω₂, ω₃.

Calculated vᵢ = −ωᵢ × qᵢ.

Constructed S₁, S₂, S₃ and the total space screw matrix.

Constructed the home transformation M = T(0,0,0).

Established the space-frame POE forward-kinematics equation.

Converted the space screws into body screws using Bᵢ = Ad(M⁻¹)Sᵢ.

Established the equivalent body-frame POE equation.

# 11. Current Results — One-Page Reference

Space screws:

S₁ = [0,0,1,0,0,0]ᵀ
S₂ = [1,0,0,0,0.30,0]ᵀ
S₃ = [1,0,0,0,0.30,0]ᵀ

Home configuration:

M = [ [1,0,0,0.70], [0,1,0,0], [0,0,1,0.30], [0,0,0,1] ]

Body screws:

B₁ = [0,0,1,0,0.70,0]ᵀ
B₂ = [1,0,0,0,0,0]ᵀ
B₃ = [1,0,0,0,0,0]ᵀ

This document intentionally stops here. Velocity kinematics, Jacobians, inverse kinematics, dynamics and control are not included.