3-DOF Robot: Velocity Kinematics and Statics

Continuation of the POE / Forward Kinematics Documentation

This document continues the previously derived Product of Exponentials (POE) and forward-kinematics work. It develops velocity kinematics, space and body Jacobians, and statics, with the manual derivations and final results.

# 1. Robot Model and Previously Derived Results

Assumed dimensions:

l1 = 0.30 m

l2 = 0.40 m

l3 = 0.30 m

Working joint chain: θ1 → l1 → θ2 → l2 → θ3 → l3 → End Effector.

Current geometric assumptions: Joint 1 is at the origin and rotates about space Z; Joint 2 is at q2=[0,0,0.30]^T and rotates about space X; Joint 3 is at q3=[0.40,0,0.30]^T and is currently assumed to rotate about space X; at home l3 extends along +X and the home end-effector orientation is aligned with the space frame.

# 2. Space Screw Axes

For a revolute joint, S=[ω;v], where v=−ω×q.

S1 = [0, 0, 1, 0, 0, 0]^T

S2 = [1, 0, 0, 0, 0.30, 0]^T

S3 = [1, 0, 0, 0, 0.30, 0]^T

Therefore S=[S1 S2 S3].

# 3. Home Pose and Body Screws

At home, the end effector is at x=l2+l3=0.70 m and z=l1=0.30 m, with R=I.

M = [[1,0,0,0.70],[0,1,0,0],[0,0,1,0.30],[0,0,0,1]]

Body screws are Bi=Ad(M^−1)Si:

B1 = [0, 0, 1, 0, 0.70, 0]^T

B2 = [1, 0, 0, 0, 0, 0]^T

B3 = [1, 0, 0, 0, 0, 0]^T

# 4. What Is Velocity Kinematics?

Forward kinematics maps joint positions θ to the end-effector pose T. Velocity kinematics maps joint velocities θ̇ to the instantaneous end-effector velocity.

Joint velocity vector: θ̇=[θ̇1, θ̇2, θ̇3]^T.

A spatial or body twist is V=[ω;v]=[ωx,ωy,ωz,vx,vy,vz]^T.

Space velocity: Vs=Js(θ)θ̇.

Body velocity: Vb=Jb(θ)θ̇.

The space twist is expressed in the fixed/world frame; the body twist is expressed in the end-effector frame.

# 5. Adjoint Transformation

For T=[[R,p],[0,1]], AdT=[[R,0],[[p]R,R]], where [p] is the skew-symmetric matrix of p.

[p]=[[0,−pz,py],[pz,0,−px],[−py,px,0]].

For a twist [ω;v]: AdT[ω;v]=[Rω; p×(Rω)+Rv].

# 6. Space Jacobian — Full Derivation

For T=e^[S1]θ1 e^[S2]θ2 e^[S3]θ3 M:

Js=[S1, Ad(T1)S2, Ad(T1T2)S3].

## 6.1 Column 1

Js1=S1=[0,0,1,0,0,0]^T.

## 6.2 Column 2

T1=e^[S1]θ1=[[c1,−s1,0,0],[s1,c1,0,0],[0,0,1,0],[0,0,0,1]], where c1=cosθ1 and s1=sinθ1.

Since p1=0, Ad(T1)=diag(Rz(θ1),Rz(θ1)).

Multiplication gives Js2=[c1,s1,0,−0.30s1,0.30c1,0]^T.

## 6.3 Column 3

For S2, T2=e^[S2]θ2=[[1,0,0,0],[0,c2,−s2,0.30s2],[0,s2,c2,0.30(1−c2)],[0,0,0,1]], where c2=cosθ2 and s2=sinθ2.

The rotation of T1T2 is R=[[c1,−s1c2,s1s2],[s1,c1c2,−c1s2],[0,s2,c2]].

Its translation is p=[−0.30s1s2, 0.30c1s2, 0.30(1−c2)]^T.

Applying Ad(T1T2) to S3 gives Js3=[c1,s1,0,−0.30s1c2,0.30c1c2,0.30s2]^T.

## 6.4 Final Space Jacobian

Js = [[0,c1,c1],[0,s1,s1],[1,0,0],[0,−0.30s1,−0.30s1c2],[0,0.30c1,0.30c1c2],[0,0,0.30s2]].

# 7. Body Jacobian — Full Derivation

For T=M e^[B1]θ1 e^[B2]θ2 e^[B3]θ3:

Jb=[Ad(e^[−B2]θ2 e^[−B3]θ3)B1, Ad(e^[−B3]θ3)B2, B3].

## 7.1 Column 3

Jb3=B3=[1,0,0,0,0,0]^T.

## 7.2 Column 2

Because B2=B3 and both are rotations about the same X-axis through the body origin, the adjoint rotation about that same axis leaves B2 unchanged.

Jb2=B2=[1,0,0,0,0,0]^T.

## 7.3 Column 1

Because B2=B3, the exponentials combine into a rotation about X by −(θ2+θ3). Let α=θ2+θ3.

Applying its adjoint to B1 gives Jb1=[0,sinα,cosα,0,0.70cosα,−0.70sinα]^T.

## 7.4 Final Body Jacobian

Jb=[[0,1,1],[sinα,0,0],[cosα,0,0],[0,0,0],[0.70cosα,0,0],[−0.70sinα,0,0]], α=θ2+θ3.

# 8. Applying the Jacobian to Velocity

Given θ̇=[θ̇1,θ̇2,θ̇3]^T:

Vs=Jsθ̇.

Vb=Jbθ̇.

The first three components are angular velocity in rad/s; the last three are linear velocity in m/s.

Example input: θ̇=[0.5,0.2,0.1]^T rad/s. Multiply the appropriate Jacobian by this vector to obtain the instantaneous end-effector twist.

# 9. What Is Statics?

Statics describes the relationship between an externally applied end-effector wrench and the joint torques required for equilibrium.

A wrench is F=[n;f]=[mx,my,mz,fx,fy,fz]^T. The first three components are moments in N·m and the last three are forces in N.

The same Jacobian used for velocity appears transposed in statics.

# 10. Why τ=J^T F?

Using instantaneous power / virtual work: F^T V = τ^T θ̇.

Since V=Jθ̇, F^T Jθ̇=τ^Tθ̇.

Therefore τ=J^T F.

Thus J maps joint velocity to end-effector twist, while J^T maps an end-effector wrench to joint torques.

# 11. Space Statics — Full Calculation

Space statics: τ=Js^T Fs.

Let Fs=[mx,my,mz,fx,fy,fz]^T.

Js^T=[[0,0,1,0,0,0],[c1,s1,0,−0.30s1,0.30c1,0],[c1,s1,0,−0.30s1c2,0.30c1c2,0.30s2]].

Multiplying row by row:

τ1 = mz.

τ2 = c1 mx + s1 my − 0.30 s1 fx + 0.30 c1 fy.

τ3 = c1 mx + s1 my − 0.30 s1 c2 fx + 0.30 c1 c2 fy + 0.30 s2 fz.

Final: τ=[mz, c1mx+s1my−0.30s1fx+0.30c1fy, c1mx+s1my−0.30s1c2fx+0.30c1c2fy+0.30s2fz]^T.

# 12. Simple Space-Statics Example

For a pure Z force, Fs=[0,0,0,0,0,fz]^T.

Then τ1=0, τ2=0, τ3=0.30 sinθ2 fz.

Therefore τ=[0,0,0.30 sinθ2 fz]^T.

# 13. Body Statics

Body statics uses the body Jacobian and a wrench expressed in the body/end-effector frame:

τ=Jb^T Fb.

Let Fb=[mx,my,mz,fx,fy,fz]^T and α=θ2+θ3.

## 13.1 Transpose

Jb^T=[[0,sinα,cosα,0,0.70cosα,−0.70sinα],[1,0,0,0,0,0],[1,0,0,0,0,0]].

## 13.2 Multiply

τ1 = sinα my + cosα mz + 0.70 cosα fy − 0.70 sinα fz.

τ2 = mx.

τ3 = mx.

Final: τ=[sin(θ2+θ3)my+cos(θ2+θ3)mz+0.70cos(θ2+θ3)fy−0.70sin(θ2+θ3)fz, mx, mx]^T.

# 14. Space vs Body Summary

| Quantity | Space | Body |
| --- | --- | --- |
| Jacobian | Js | Jb |
| Velocity | Vs=Jsθ̇ | Vb=Jbθ̇ |
| Wrench | Fs in space frame | Fb in body frame |
| Statics | τ=Js^T Fs | τ=Jb^T Fb |
| Reference frame | Fixed/world | End effector/body |

Both formulations describe the same physical robot. Their components differ because they are expressed in different coordinate frames. Consistent frame transformations give the same physical joint torques.

# 15. Complete Workflow

1. Define joint axes and points qi.

2. Construct space screws Si=[ωi;vi].

3. Determine the home pose M.

4. Convert to body screws: Bi=Ad(M^−1)Si.

5. Forward kinematics: T=e^[S1]θ1 e^[S2]θ2 e^[S3]θ3 M.

6. Space Jacobian: Js=[S1,Ad(T1)S2,Ad(T1T2)S3].

7. Body Jacobian: Jb=[Ad(e^[−B2]θ2e^[−B3]θ3)B1,Ad(e^[−B3]θ3)B2,B3].

8. Velocity: Vs=Jsθ̇ and Vb=Jbθ̇.

9. Space statics: τ=Js^T Fs.

10. Body statics: τ=Jb^T Fb.

# 16. Modeling Note

All calculations above follow the current assumed robot model used in the POE/FK work. This model produces S2=S3 and B2=B3. Because identical screw axes for two distinct physical joints is unusual, the Joint 3 axis should be checked against the original schematic before using these numerical Jacobian/statics equations for a real robot or controller. The derivation here deliberately preserves the current model.

# 17. Final Results

Velocity: Vs=Jsθ̇, Vb=Jbθ̇.

Statics: τ=Js^T Fs, τ=Jb^T Fb.

Space Jacobian: Js=[[0,c1,c1],[0,s1,s1],[1,0,0],[0,−0.30s1,−0.30s1c2],[0,0.30c1,0.30c1c2],[0,0,0.30s2]].

Body Jacobian: Jb=[[0,1,1],[sinα,0,0],[cosα,0,0],[0,0,0],[0.70cosα,0,0],[−0.70sinα,0,0]], α=θ2+θ3.

Space statics: τ1=mz; τ2=c1mx+s1my−0.30s1fx+0.30c1fy; τ3=c1mx+s1my−0.30s1c2fx+0.30c1c2fy+0.30s2fz.

Body statics: τ1=sinα my+cosα mz+0.70cosα fy−0.70sinα fz; τ2=mx; τ3=mx.